# Lumen T2 Label Printer

Quick Start Guide

Contents

What’s in the box	2

Setting up	2

Printing your first label	3

Specifications	4

Status light	5

# What’s in the box

- ☒ Lumen T2 printer

- ☒ USB-C cable (1 m)

- ☒ Power adapter, 24 V

- ☒ One roll of 50 × 30 mm labels

Warning

Use only the power adapter supplied with the printer. Other adapters can damage the print head.

# Setting up

- Open the top cover by pressing the release button on the right side.

- Load the label roll with the labels facing up, and pull about 5 cm through the slot.

- Close the cover until it clicks. The printer feeds one label to find the gap.

- Connect the power adapter and press the power button for two seconds.

Figure 1. The status light during setup

# Printing your first label

Install the Lumen app, then add the printer:

Settings → Printers → Add → Lumen T2

Tip

Hold the feed button for three seconds to print a test label with the printer’s settings.

# Specifications

Table 1. Lumen T2 specifications

| Item | Value |
| --- | --- |
| Print method | Direct thermal |
| Resolution | 300 dpi |
| Print speed | up to 150 mm/s |
| Label width | 20–60 mm |
| Connection | USB-C, Wi-Fi 2.4 GHz |
| Size | 118 × 96 × 82 mm |

# Status light

| Light | Meaning | What to do |
| --- | --- | --- |
| Green | Ready | — |
| Blinking green | Printing | Wait |
| Orange | Out of labels or cover open | Load labels, close the cover |
| Red | Print head too hot | Pause for five minutes |